17: WATER MANAGEMENT & HYDRAULIC ENGINEERING

Anicuts, Stone Aqueducts, Siphons, Stepped Tanks, Canals and Agricultural Networks

| Project: | Antigravity Indian Cultural Heritage Knowledge Platform (Hampi, Karnataka) |
| --- | --- |
| Research Team: | Historians, Epigraphists, Archaeologists, Architectural Historians, Anthropologists, Digital Knowledge Architects |
| Source Hierarchy: | Tier 1 (UNESCO, ASI, State Epigraphy) & Tier 2 (Academic Monographs & Journals) |
| Geographical Context: | Hampi World Heritage Site, Vijayanagara Metropolitan Region, Bellary/Vijayanagara Dist, Karnataka |

1. Master Hydraulic System of Vijayanagara

To sustain an imperial metropolis of an estimated 500,000 residents within the semi-arid Deccan plateau, Vijayanagara engineers developed an extraordinary hydraulic network that integrated perennial river diversions, stone aqueducts, storage reservoirs, and subterranean conduits.

Major Canals & Tungabhadra Masonry Dams (Anicuts)

| Canal / Anicut Name | Period / Patron | Length / Capacity | Function & Irrigated Area |
| --- | --- | --- | --- |
| Turtha Canal & Dam | Devaraya I (c. 1410 CE) | 18 km masonry channel | Diverts Tungabhadra waters along the base of Matanga Hill to irrigate the Sacred Centre valley; still active today. |
| Raya Canal | Krishnadevaraya (1521 CE) | 24 km gravity canal | Supplied the royal suburban agricultural zones around Kamalapuram and Hospet. |
| Basavanna Canal | 15th Century Sangama | 28 km stone canal | One of the longest channels, feeding fertile banana, sugarcane, and paddy terraces. |
| Kamalapuram Tank | Early 15th Century | 1.5 sq km water reservoir | Massive earthen and stone bund reservoir holding ~10 million cubic meters; fed royal urban aqueducts. |
| Anegundi Canal | 14th Century | 15 km north-bank canal | Irrigated the royal orchards and paddy fields of Anegundi citadel. |

2. Advanced Engineering Techniques

• Gravity-Fed Stone Aqueducts: Elevated monolithic granite U-shaped channels joined with water-tight lime mortar and supported on stone pylons.
• Inverted Siphon Technology: Used to transport water across undulating depressions without mechanical pumping.
• Sedimentation & Filtration Chambers: Silt-trapping settling basins installed before water entered royal baths and temple pushkaranis.
• Earthen Embankments & Sluices: Massive granite-faced earthen bunds with stone-carved sluice gates operated by wooden plugs (Madagu).