58: THREATS, VULNERABILITIES & HERITAGE CHALLENGES

Mass Tourism Pressures, Climate Weathering, Hydraulic Flooding, and Modern Development Risks

| Project: | Antigravity Indian Cultural Heritage Knowledge Platform (Hampi, Karnataka) |
| --- | --- |
| Research Team: | Historians, Epigraphists, Archaeologists, Architectural Historians, Anthropologists, Digital Knowledge Architects |
| Source Hierarchy: | Tier 1 (UNESCO, ASI, State Epigraphy) & Tier 2 (Academic Monographs & Journals) |
| Geographical Context: | Hampi World Heritage Site, Vijayanagara Metropolitan Region, Bellary/Vijayanagara Dist, Karnataka |

1. Threat Matrix Assessment

As a fragile open-air archaeological park, Hampi faces multiple ongoing environmental, anthropogenic, and structural threats:

| Threat Category | Specific Risk Factor | Potential Impact | Mitigation & Monitoring Strategy |
| --- | --- | --- | --- |
| Anthropogenic / Tourism | Unregulated foot traffic, touching delicate carvings | Micro-abrasion and wear on musical pillars and reliefs | Installation of wooden boardwalks, glass barriers, and security personnel. |
| Environmental / Climate | Granite thermal exfoliation, acid rain, lichens | Chemical biodeterioration and micro-cracking of stone | Application of biocidal washes and breathable ethyl silicate stone consolidants. |
| Hydrological / River | Monsoon dam water discharge from Tungabhadra Dam | Periodic flooding of riverside monuments (Purandaradasa mandapa) | Real-time flood warning telemetry and stone foundation reinforcement. |
| Urbanization & Traffic | Heavy vehicular vibration on access roads | Foundation settlement of ancient gateways and gopurams | Diversion of heavy commercial traffic away from core heritage roads. |