CHANDERI MADHYA PRADESH — CULTURAL HERITAGE & KNOWLEDGE BASE

23. Chanderi Weaving Process — Step by Step

1. Overview of the Production Process

The production of Chanderi fabric is a multi-stage artisanal process requiring specialized knowledge at every step. In traditional practice, different stages may be performed by different family members or specialist workers within the weaver community. The following describes the traditional handloom production process [Source: CHA-S005, CHA-S012].

2. Stage-by-Stage Process

| Stage | Process | Traditional Method | Materials | Key Skill |
| --- | --- | --- | --- | --- |
| 1. Raw Material Selection | Selection of yarn quality | Visual and tactile inspection of silk and cotton yarn. | Katan silk (from Bangalore/Mysore or local suppliers); fine cotton yarn. | Yarn quality assessment; experience-based selection. |
| 2. Yarn Sourcing | Obtaining raw silk and cotton | Traditionally sourced from specialized yarn markets; modern: government supply depots, cooperatives. | Katan silk; fine-count cotton. | Market knowledge; quality judgment. |
| 3. Degumming (Silk) | Removing sericin gum from raw silk | Boiling silk in mild alkali (soap water) solution to remove natural gum and achieve softness. | Raw silk; soap; water. | Temperature control; timing. |
| 4. Dyeing | Applying colour to yarn | Traditional: natural dyes (indigo, turmeric, lac); Modern: synthetic acid dyes or reactive dyes for silk. | Dye materials; mordants (alum, iron). | Colour chemistry; evenness of dye uptake. |
| 5. Winding (Reeling) | Converting skein yarn to bobbins/pirns | Hand winding or charkha (spinning wheel) winding. | Yarn skeins; wooden bobbins. | Even tension; no tangles. |
| 6. Warping | Creating the warp (longitudinal threads) | Warping board or warping mill used to measure and arrange warp threads to exact length and count. | Silk or cotton warp yarn. | Precision thread count; length accuracy. |
| 7. Sizing (Starch) | Stiffening warp yarn for weaving tension | Rice-starch or maida (wheat starch) solution applied to warp thread for controlled tension. | Rice starch; water. | Even application; correct concentration. |
| 8. Loom Dressing | Threading the loom | Each warp thread passed through heddle eye and reed dent individually — extremely labor-intensive. | Warp beam; heddles; reed. | Precision threading; pattern knowledge. |
| 9. Weaving | Interlacing warp and weft on the handloom | Pit loom or frame loom. Weaver operates treadles with feet, throws shuttle with hand, beats weft with beater. | Weft thread (cotton/silk); shuttle; bobbin. | Rhythm; tension consistency; motif execution. |
| 10. Motif Weaving | Creating woven-in design motifs | Supplementary weft technique: extra weft threads manually inserted to create motif pattern row by row. | Supplementary weft (silk or zari). | Pattern memory; manual dexterity. |
| 11. Finishing | Post-weaving treatment | Washing to remove sizing; starching/softening as required; pressing. | Water; starch; iron. | Careful handling of delicate fabric. |
| 12. Quality Inspection | Checking completed fabric | Visual inspection for missing threads, motif errors, uneven density. | Completed fabric. | Expert eye; quality standards knowledge. |

[PROCESS DOCUMENTATION NOTE] The Chanderi weaving process is documented here based on published Ministry of Textiles and Handloom Development sources. Individual weavers may follow variant methods passed down within families. The process described represents the general traditional technique rather than any single household's exact practice. [Source: CHA-S005, CHA-S012]